# ABCD Efficiency Validation

This document reproduces the current GUI calculation path for the ABCD-based efficiency mode using the requested CSV input and the requested table position.

## 1. Input selection

| Item | Value |
| --- | --- |
| CSV input file | 43200112_PB 663 LF_full Smap template.csv |
| Cable de-embed source used by GUI | default (no cable) |
| Requested C1 percentage | 50.56% |
| Requested C2 percentage | 6.04% |
| Grid denominator for C1 | 447 |
| Grid denominator for C2 | 447 |
| Chosen X_C1 index | 226 |
| Chosen Y_C2 index | 27 |
| Actual displayed C1 percentage | 50.559284116331% |
| Actual displayed C2 percentage | 6.040268456376% |
| C1 coarse / fine | 3 / 34 |
| C2 coarse / fine | 0 / 27 |
| Frequency | 430000 Hz |

## 2. S-parameters at the selected point

The GUI first parses the CSV, then applies the default 'no cable' de-embed path before using the data for the Efficiency tab.

| Item | Value |
| --- | --- |
| Raw CSV S11 | 0.116076 - 0.683203j |
| Raw CSV S21 | -0.148091 - 0.68489j |
| Raw CSV S12 | -0.149445 - 0.68426j |
| Raw CSV S22 | 0.376644 + 0.510978j |
| GUI-path S11 | 0.116077311289 - 0.683202913119j |
| GUI-path S21 | -0.148089751251 - 0.684889793304j |
| GUI-path S12 | -0.149443751409 - 0.684259795997j |
| GUI-path S22 | 0.376643132392 + 0.510977730439j |

## 3. Load impedance from S22

The current implementation uses the selected point's S22 as the load reflection coefficient and converts it with ZL = Z0 * (1 + ΓL) / (1 - ΓL), where Z0 = 50 ohm.

| Item | Value |
| --- | --- |
| Gamma_L | 0.376643132392 + 0.510977730439j |
| ZL | 45.9494703848 + 78.6516442854j |

## 4. Z-parameters from the 2-port S-matrix

The code converts S to Z using the existing s_to_z_parameter_values() function.

| Item | Value |
| --- | --- |
| Z11 | 1.26812873466 - 29.2090610495j |
| Z12 | -2.37005509225 - 51.2172957126j |
| Z21 | -2.26459654217 - 51.24610888j |
| Z22 | 5.38665446699 + 60.1731240471j |

## 5. ABCD matrix from the Z-matrix

The code converts Z to ABCD with A = Z11 / Z21, B = (Z11*Z22 - Z12*Z21) / Z21, C = 1 / Z21, D = Z22 / Z21.

| Item | Value |
| --- | --- |
| A | 0.567773885043 + 0.0498361254597j |
| B | 2.42966146704 + 85.6504741159j |
| C | -0.000860640506443 + 0.0194756444596j |
| D | -1.17654634299 + 0.0531211392717j |

## 6. Voltage and current calculation

The requested method assumes IL = 1 Arms. Then V2 = ZL * IL, V1 = A*V2 + B*IL, and I1 = C*V2 + D*IL.

| Item | Value |
| --- | --- |
| IL | 1 + 0j |
| V2 | 45.9494703848 + 78.6516442854j |
| V1 | 24.5988775709 + 132.596767328j |
| I1 | -2.74788377872 + 0.880325896623j |
| Zin = V1 / I1 | 5.90132859356 - 46.3635601799j |

## 7. Power and efficiency

The current implementation uses PL = Re(ZL) * |IL|^2 and Pin = Re(Zin) * |I1|^2, then efficiency = PL / Pin. For reference, Re(V * conj(I)) is also listed and should match.

| Item | Value |
| --- | --- |
| PL from Re(ZL) * ¦IL¦^2 | 45.9494703848 |
| PL from Re(V2 * conj(IL)) | 45.9494703848 |
| Pin from Re(Zin) * ¦I1¦^2 | 49.1335114353 |
| Pin from Re(V1 * conj(I1)) | 49.1335114353 |
| Efficiency = PL / Pin | 0.935196142968 |
| Displayed table value | 0.9352 |

## 8. Reproducibility note

This document was generated by running the current MatchResolution.py code path directly against the selected CSV file. If the numbers differ from expectation, the mismatch can now be checked step-by-step: input row, S22-to-ZL, S-to-Z, Z-to-ABCD, or power formula.