# ABCD Efficiency Validation

This document recalculates the ABCD-based efficiency at the requested CSV point using a fixed load impedance ZL = 43 - j82 ohm and IL = 1 Arms.

## 1. Input selection

| Item | Value |
| --- | --- |
| CSV input file | 43200112_PB 663 LF_full Smap template.csv |
| Cable de-embed source used by GUI | default (no cable) |
| Requested C1 percentage | 50.56% |
| Requested C2 percentage | 6.04% |
| Grid denominator for C1 | 447 |
| Grid denominator for C2 | 447 |
| Chosen X_C1 index | 226 |
| Chosen Y_C2 index | 27 |
| Actual displayed C1 percentage | 50.559284116331% |
| Actual displayed C2 percentage | 6.040268456376% |
| C1 coarse / fine | 3 / 34 |
| C2 coarse / fine | 0 / 27 |
| Frequency | 430000 Hz |

## 2. S-parameters at the selected point

The GUI first parses the CSV, then applies the default 'no cable' de-embed path before using the data for the Efficiency tab.

| Item | Value |
| --- | --- |
| Raw CSV S11 | 0.116076 - 0.683203j |
| Raw CSV S21 | -0.148091 - 0.68489j |
| Raw CSV S12 | -0.149445 - 0.68426j |
| Raw CSV S22 | 0.376644 + 0.510978j |
| GUI-path S11 | 0.116077311289 - 0.683202913119j |
| GUI-path S21 | -0.148089751251 - 0.684889793304j |
| GUI-path S12 | -0.149443751409 - 0.684259795997j |
| GUI-path S22 | 0.376643132392 + 0.510977730439j |

## 3. Fixed load impedance

For this validation run, the load is forced to ZL = 43 - j82 ohm. It is not derived from S22.

| Item | Value |
| --- | --- |
| Fixed ZL | 43 - 82j |
| Equivalent Gamma_L for reference | 0.39504325766 - 0.533402719053j |

## 4. Z-parameters from the 2-port S-matrix

The code converts S to Z using the existing s_to_z_parameter_values() function.

| Item | Value |
| --- | --- |
| Z11 | 1.26812873466 - 29.2090610495j |
| Z12 | -2.37005509225 - 51.2172957126j |
| Z21 | -2.26459654217 - 51.24610888j |
| Z22 | 5.38665446699 + 60.1731240471j |

## 5. ABCD matrix from the Z-matrix

The code converts Z to ABCD with A = Z11 / Z21, B = (Z11*Z22 - Z12*Z21) / Z21, C = 1 / Z21, D = Z22 / Z21.

| Item | Value |
| --- | --- |
| A | 0.567773885043 + 0.0498361254597j |
| B | 2.42966146704 + 85.6504741159j |
| C | -0.000860640506443 + 0.0194756444596j |
| D | -1.17654634299 + 0.0531211392717j |

## 6. Voltage and current calculation

The requested method assumes IL = 1 Arms. Then V2 = ZL * IL, V1 = A*V2 + B*IL, and I1 = C*V2 + D*IL.

| Item | Value |
| --- | --- |
| IL | 1 + 0j |
| V2 | 43 - 82j |
| V1 | 30.9305008116 + 41.2359689371j |
| I1 | 0.383448960917 + 0.961146372563j |
| Zin = V1 / I1 | 48.0877525241 - 12.9962536716j |

## 7. Power and efficiency

The current implementation uses PL = Re(ZL) * |IL|^2 and Pin = Re(Zin) * |I1|^2, then efficiency = PL / Pin. For reference, Re(V * conj(I)) is also listed and should match.

| Item | Value |
| --- | --- |
| PL from Re(ZL) * ¦IL¦^2 | 43 |
| PL from Re(V2 * conj(IL)) | 43 |
| Pin from Re(Zin) * ¦I1¦^2 | 51.4940703599 |
| Pin from Re(V1 * conj(I1)) | 51.4940703599 |
| Efficiency = PL / Pin | 0.835047602558 |

## 8. Reproducibility note

This document was generated by running the current MatchResolution.py parsing and matrix-conversion code against the selected CSV file, then replacing the load with the fixed user-requested ZL = 43 - j82 ohm. If the numbers differ from expectation, the mismatch can now be checked step-by-step: input row, S-to-Z, Z-to-ABCD, or power formula.