# MatchResolution Executable User Manual

Version: v1.0.6
Audience: End users running the Windows executable only (no Python setup required).

Document date: 2026-07-18

# 1. Start the App

1. Double-click: dist\MatchResolution_V1.0.6.exe

2. The app opens maximized.

# 2. Quick Workflow

1. Click "Browse File" and select your measurement file (.csv/.txt/.dat/.s2p/.log).

2. (Optional) Click "Browse Cable File" and select a cable S-parameter file.

3. Click "Convert".

4. Review results in the tabs (Display, X-Y Table, Impedance, dZ, Reflect Coefficient, Efficiency, Smith Chart, Component).

5. Click "Export CSV" to export the current tab data.

# 3. Cable File (Optional)

If no cable file is selected, the app uses built-in default no-cable values automatically.

Cable file must include these columns and both Cable1/Cable2 rows:

cable, S11R, S11X, S21R, S21X, S12R, S12X, S22R, S22X

| cable | S11R | S11X | S21R | S21X | S12R | S12X | S22R | S22X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cable1 | 0.016788 | 0.004683 | 0.67576 | -0.71964 | 0.67565 | -0.71961 | 0.018742 | 0.004242 |
| Cable2 | 0.019153 | 0.001615 | 0.612062 | -0.77358 | 0.612133 | -0.77335 | 0.017781 | 0.00219 |

# 4. What Convert Does

During conversion, the app de-embeds cable effects using Cable1/Cable2 by:

- Converting S parameters to T parameters

- Removing cable effects in T-domain

- Converting final T parameters back to S parameters

The Display table and all plots use the de-embedded final results.

# 5. Tab Guide

Display: Row-based converted/de-embedded S-parameter table.

X-Y Table: Grid view for selected S-parameter (S11/S21/S12/S22).

Impedance: Converted impedance view for selected parameter.

dZ: Delta impedance resolution maps.

Reflect Coefficient: Delta-Gamma resolution views.

Efficiency: |S21|^2 or ?overall = (1 - |S11|^2) ? |S21|^2.

Smith Chart: Plot modes (X-Y, dZ, d?, Efficiency), hover info, and ZL search.

Component: Capacitor and resolution component plots.

# 6. Smith Chart Search and Carry Over

1. Enter C1% and C2%.

2. Click "Search ZL" (located under C1%).

3. Click "Carry Over" (located under C2%) to add the result to manual points.

# 7. Export CSV

Click "Export CSV" to export data from the currently selected tab.

# 8. Troubleshooting

No data after Convert: Confirm the input file contains valid Frequency/CMD/S-parameter data.

Cable file error: Confirm required columns and both Cable1/Cable2 rows are present.

Search ZL not available: Convert data first, then run Search ZL before Carry Over.

# 9. Best Practices

Keep original measurement files unchanged.

Use a cable file only when measurements include front/back cable effects.

Export each tab result needed for reporting.