S-parameter to Z-parameter Conversion Guide

For the Zpar tab in MatchResolution

# 1. Assumptions

This conversion assumes a 2-port network.

Both ports use the same reference impedance Z0 (the app uses 50 ohms).

The S-parameters are complex values at each X_C1 / Y_C2 grid point.

# 2. Matrix formula

The correct conversion is:

Z = Z0 * (I + S) / (I - S)

where S and Z are 2x2 complex matrices and I is the 2x2 identity matrix.

# 3. Element-by-element formulas

Let:

Δ = (1 - S11)(1 - S22) - S12S21

Z11 = Z0 * [ (1 + S11)(1 - S22) + S12S21 ] / Δ

Z12 = Z0 * [ 2S12 ] / Δ

Z21 = Z0 * [ 2S21 ] / Δ

Z22 = Z0 * [ (1 - S11)(1 + S22) + S12S21 ] / Δ

# 4. How the table is built

Read S11, S21, S12, and S22 at each grid location.

Compute Δ for that row.

If Δ is too close to zero, the conversion is numerically unstable and should be skipped.

Otherwise compute Z11, Z12, Z21, and Z22 and place the selected value into the Zpar grid.

# 5. Important note

You cannot convert a 2-port S matrix to a correct Z matrix from only S22. The full S matrix is required because each Z element depends on all four S-parameters.

# 6. Practical check

If you only need load impedance from a reflection coefficient, use Z = Z0(1 + ?)/(1 - ?). That is a different calculation from the full 2-port S-to-Z conversion above.